Report: HPC Fluid Dynamics Simulation (LBM)

Student: Ivana Radovanović

Index: SV 23/2022

System: Lenovo IdeaPad Gaming 3 (AMD Ryzen 7 5800H, 8 Cores / 16 Threads, 16GB RAM)

1. Technical specification and algorithm

The project implements 2D fluid simulation using the Lattice Boltzmann method (LBM) with the D2Q9 model. The algorithm models the movement of fluids through discrete functions of particle distribution in nine directions.

1.1. Key steps of the algorithm

- Collision Step: Local relaxation of particles towards equilibrium (BGK operator).

- Streaming Step: Advection of distribution to neighboring nodes of the network.

- Boundary Conditions: A "bounce-back" (No-slip) condition for modeling solid obstacles (cylinder) and periodic conditions for simulating a continuous channel have been implemented.

- Numerical stability:  The relaxation parameter τ=0.6 was used for the simulation. This parameter directly affects the viscosity of the fluid and the stability of the BGK operator. A value close to 0.5 allows the simulation of low-viscosity fluids, but increases the risk of numerical divergence at higher speeds.

- D2Q9 Discretization: The selection of the D2Q9 model (2 dimensions, 9 velocity vectors) provides an optimal balance between computational complexity and accuracy for the simulation of incompressible flow in channels.

2. Memory Parallelization and Optimization Strategy

The main focus of the implementation was on eliminating "bottlenecks" in memory access, which is crucial for modern multi-core processors.

2.1. Rust (Rayon & Zero-copy)

- Domain Decomposition: Domains are divided into blocks of rows, which maximizes the utilization of the L1/L2 cache (Spatial Locality).

- Loop Fusion: Streaming and Collision are merged into a single pass through the matrix, halving the number of reads from RAM.

- Pointer Swapping: The use of two buffers and the std::mem::swap function eliminates the need to allocate and clone memory in each iteration.

- Memory Locality and Prefetching: By using axis_iter_mut(Axis(1)) in combination with Rayon, it is ensured that threads process data in an order that is aligned with the memory layout (row-major order). This allows the processor to make efficient use of hardware prefetching mechanisms, reducing the waiting time for data from RAM.

- No-lock Policy: The implementation relies on the "Pull" streaming model where each thread reads from the old state and writes to the new buffer without the need for synchronization primitives such as Mutex or RwLock, thus avoiding contention at the memory level.

2.2. Python (Shared Memory)

- RawArray: Shared memory over multiprocessing is implemented. RawArray, which avoids slow IPC (Inter-process communication) string copying costs.

- Barrier Synchronization: The use of barriers ensures that processes transition between calculation phases in a synchronized manner.

- Elimination of GIL constraints: By using a multiprocessing module instead of threading, the Global Interpreter Lock (GIL) was bypassed, allowing collision steps to be executed in full parallel on all physical cores.

- Zero-Copy View: Connecting RawArray memory to numpy arrays using np.frombuffer enabled the application of fast vector operations without additional copying of data within worker processes.

3. Performance and scaling analysis

Testing was performed with 30 independent samples per configuration to ensure statistical significance.

3.1. Strong Scaling (Amdahl analysis)

A test on a fixed grid (400x100) examines how much the increase in threads speeds up a fixed problem.

| Threads | Mean time (s) | Acceleration (Speedup) | Efficiency |
| --- | --- | --- | --- |
| 1 | 4.1316 | 1.00x | 100% |
| 4 | 1.2940 | 3.19x ($4.13 / $1.29) | 79.8% |
| 8 | 0.9499 | 4.35x ($4.13 / $0.95) | 54.4% |
| 16 | 0.9246 | 4.47x ($4.13 / $0.92) | 27.9% |

- Efficiency Analysis: The drop in efficiency from 100% to 27.9% when switching from 1 to 16 threads indicates that the cost of running threads and synchronization has become dominant compared to the relatively low computational weight of a 400x100 network.

- Impact of SMT (Hyper-threading): A minimum gain of between 8 and 16 threads confirms that the LBM algorithm, as a memory-intensive operation, saturates the memory bandwidth already with physical cores, and logical cores (threads) cannot provide significant acceleration because they share the same path to memory.

Analysis: The maximum acceleration is achieved at 8 threads, which corresponds to the number of physical cores of the Ryzen 7 processor. The small difference between 8 and 16 threads (0.95s vs. 0.92s) confirms that the simulation is limited by memory bandwidth rather than the number of logic cores (SMT).

3.2. Weak Scaling (Gustafson analysis)

A test where the volume of work increases in proportion to the number of threads (200 x threads).

| Threads | Dimensions (WxH) | Mean time (s) | Standard deviation (Std Dev) |
| --- | --- | --- | --- |
| 1 | 200 x 100 | 2.2289 | 0.0496 |
| 2 | 400 x 100 | 1.9830 | 0.0100 |
| 4 | 800 x 100 | 2.3716 | 0.0341 |
| 8 | 1600 x 100 | 3.1108 | 0.0300 |
| 16 | 3200 x 100 | 6.9590 | 0.1029 |

- System Stability: A noticeable increase in Standard Deviation (from 0.05s to 0.10s) when using 16 threads indicates stable system operation even at extremely wide 3200-pixel meshes.

- Memory scalability: The fact that the time increase factor (3.12x, obtained as 6.95/2.22) is significantly smaller than the workload increase factor (16x) confirms that the implementation is 'weakly scalable'.

Analysis: Although the network on 16 threads is as much as 16 times larger than on one, the execution time has increased only 3.12 times. This demonstrates the excellent scalability of the system and confirms that the optimization of the "Pull" streaming model effectively suppresses synchronization costs.

4. Comparative Analysis: Rust vs Python

Based on parallel measurements on 16 threads/processes:

- Rust Parallel: 0.92 s

- Python Parallel (Shared Memory): 2.19 s

Conclusion: Rust is ~2.4x faster than optimized Python in parallel mode. Although both versions use parallelization, Python lags behind due to the cost of object management and dynamic type checking during each iteration within  the worker process. The rust version, compiled with the --release flag, uses LLVM optimizations that aggressively unroll loops and use the processor's SIMD instructions. The advantage of Rust lies in the LLVM optimizations and the absence of the "overhead" that Python has even with shared memory.

5. Hardware Specification and Architecture Impact

For benchmarking purposes, we used a Lenovo IdeaPad Gaming 3 15ACH6 laptop with the following components:

- Processor: AMD Ryzen 7 5800H (Zen 3 architecture).

- Core configuration: 8 physical cores and 16 logic threads (SMT).

- Clock speed: Base frequency of 3.2 GHz with "boost" capability up to 4.4 GHz.

- Memory: 16GB DDR4 RAM.

The Impact of Architecture on the Results:

- L3 Cache: The Zen 3 architecture uses a unified L3 cache (16MB) that is accessed by all 8 cores within a single complex (CCX). This is crucial for a Rust Rayon implementation, as the exchange of data between threads during the work-stealing process is extremely fast, which is seen in stable times for Strong Scaling.

- Memory Bandwidth: As the LBM algorithm is highly dependent on memory bandwidth (memory-bound), the speed of DDR4 memory is the main factor that caused performance saturation after 8 threads. Additional logic threads (SMTs) could not contribute significantly because the physical cores were already making maximum use of the available memory bandwidth to read and write 3D matrices.

- Thermal scaling: Given that testing lasted through 30 consecutive samples, a small standard deviation (below 0.05s for most Rust tests) confirms that the laptop's cooling system successfully maintains a stable CPU clock speed without significant thermal throttling.

6. Conclusion

This project successfully demonstrates the power and necessity of HPC principles in modern scientific simulations. Through a comparison of Rust and Python, it has been proven that the choice of language and memory management strategies directly dictate the limits of the system's scalability.

Key contributions and observations:

- Memory optimization: The implementation of techniques such as Loop Fusion and Pointer Swapping in Rust, as well as the use  of the Shared Memory (RawArray) model in Python, has proven to be crucial for overcoming memory bottlenecks.

- Confirmation of the law of scaling: The results of the Strong Scaling test showed an acceleration of 4.47x to 16 threads, while Weak Scaling confirmed Gustafson's law – despite an increase in the volume of work of 16 times, the execution time increased only 3.12 times.

- The superiority of Rust: The Rust Parallel implementation outperformed the optimized Python by ~2.4 times, confirming the effectiveness of LLVM optimizations and Rayon'  s work-stealing scheduler on the Zen 3 architecture.

- Numerical and physical accuracy: The use of the relaxation parameter τ=0.6  provided a stable simulation, which was visually confirmed through heat maps of the flow where stagnation zones and the Venturi effect of acceleration around a cylindrical obstacle are clearly visible.

The final results confirm that stable scaling on a multi-core architecture has been achieved, thus meeting all the requirements of the technical specification while achieving high efficiency in both software ecosystems.